Faculty of Engineering, Architecture and Science

Department of Mechanical, Industrial, and Mechatronics Engineering

| Program | Electrical Engineering |
| --- | --- |
| Course Number | ELE846 |
| Course Title | Power Systems Protection Control |
| Semester/Year | F2025 |
| Instructor | Dr. Richard Cheung |

| Lab/Tutorial Report NO. | Lab Report #1 |
| --- | --- |

| Report Title | ELE846 Lab Project 1: Protection Systems Design and Verification |
| --- | --- |

| Section No. | 01 |
| --- | --- |
| Submission Date | February 13, 2025 |
| Due Date | February 17, 2025 |

| Name | Student ID | Signature* |
| --- | --- | --- |
| Shai Mourad | 501170415 |  |
| Mashal Joyaa | 500916031 |  |

*By signing above you attest that you have contributed to this submission and confirm that all

work you have contributed to this submission is your own work. Any suspicion of copying or

plagiarism in this work will result in an investigation of Academic Misconduct and may result

in a “0” on the work, an “F” in the course, or possibly more severe penalties, as well as a

Disciplinary Notice on your academic record under the Student Code of Academic Conduct,

which can be found online at:

https://www.torontomu.ca/senate/policies/pol60.pdf.

## Introduction

Protection in power systems is extremely important when considering safety in the workplace, or when determining stability and continuity of the system. Three phase, single line-to-line, and double-line-to-ground faults can cause extreme damage if not detected and cleared properly. Relays and circuit breakers are critical when isolating faulted sections or minimizing impact on the rest of the design.

This lab project focuses on the protection of two different types of power system configurations. First is a meshed transmission network, where directional overcurrent/impedance protection are created for multiple transmission lines and breakers. The second design is a radial distribution network in which overcurrent protection schemes are created. Using the provided system data, relay characteristics, and standard CT and VT ratios, detailed relay settings are calculated to ensure selectivity, sensitivity, and coordination.

By analyzing the fault scenarios at various busses, line midpoints or terminals, the design is verified and tested. From this, the project will reinforce concepts of power systems protection and control from practical experience to ensure proper use in real-world power networks.

## Results

Figure: Load Flow Results for “Figure 2” on the Lab Manual

Figure: Bus 4 Fault Results for “Figure 2” on the Lab Manual

Figure: Bus 5 Fault Circuit Results for “Figure 2” on the Lab Manual

Figure: Bus 5 Fault Results for “Figure 2” on the Lab Manual

## Conclusion

Protection systems were properly designed and implemented in this project, which verified meshed transmission and radial distribution networks. Directional overcurrent and impedance protection was properly created for transmission lines, and coordinated overcurrent protection was properly used for the radial system. Relay settings were determined and calculated based on given parameters.

The results for multiple fault locations and fault types confirmed that the protection schemes operate selectively and reliably. The faulted sections were properly isolated by the corresponding breakers, and no unnecessary tripping occurred. The results prove proper protection results in meeting requirements of speed, selectivity, and reliability.

This lab gave amazing insight into power system protection with designs and coordination. The importance of proper system modeling and analysis creates dependable system performance, further solidifying protection importance in real-world systems.

## References

[1] Department of Electrical Engineering, “ELE846 Lab Project 1: Protection Systems Design and Verification,” Lab Manual, Jan. 2026.

## Appendix (Calculations)